# Combined datasets for article

# ASYNCHRONOUS DUNE DEVELOPMENT ON A DUTCH URBANIZED BEACH DUE TO BUILDINGS AND OTHER ANTHROPOGENIC INFLUENCES

Sander Vosa,d, Christa van IJzendoornb, Roderik Lindenberghc and Alain de Wulfd

Metadata Documentation 21.12.2023

Affiliations

a	Delft University of Technology, Faculty of Civil Engineering, Department of Hydraulic Engineering, Stevinweg 1, 2628CN Delft, The Netherlands.

b	Oregon State University, College of Earth, Ocean, and Atmospheric Sciences, 104 CEOAS Admin Building, Corvallis, OR 97331-5503, USA

c	Delft University of Technology, Department of Geoscience and Remote Sensing, Stevinweg 1, 2628 CN Delft, The Netherlands.

d	Ghent University, Department of Geography, Krijgslaan 281 S8, 9000 Ghent, Belgium;

Corresponding author: s.e.vos@tudelft.nl.

| Department of hydraulic Engineering Faculty of Civil Engineering and Geoscience Delft University of Technology |  | Department of geoscience and remote sensing Faculty of Civil Engineering and Geoscience Delft University of Technology |
| --- | --- | --- |

# General Information

This data descriptor describes the data provided for the article ‘Asynchronous dune development on a Dutch urbanized beach due to buildings and other anthropogenic influences’ submitted to Geomorphology (Vos et al., 2024). The document contains an abstract of the article and an overview of the available data.

# Abstract article

Coastal dunes provide an important role to society by fulfilling various ecosystem functions. These are however under pressure due to sea level rise, climate change and urbanization. Shoreward sand transport can partly mitigate sea level rise and climate effects by contributing to dune growth but increasing urbanization of beaches can block this transport and restoration of ecosystem functions. In this paper, we investigate the long-term effects of buildings and other anthropogenic influences on dune development up to a decadal temporal scale.

A series of three topographic datasets is used to study the effect of anthropogenic actors on local dune development at an urban beach in Noordwijk, the Netherlands. Datasets range from a 100 to 3000-meter spatial scale and from a weekly to yearly temporal scale. On the smallest spatio-temporal scale topographic measurements of the effects of two containers placed on Noordwijk beach are studied. The intermediate dataset is obtained from the 2-year CoastScan project monitoring surface elevation around one beach pavilion at (bi) monthly intervals. Finally, 15 years of annual airborne lidar data along a 2.7-kilometer stretch of the beach/dune system in Noordwijk is used to evaluate the effect of 17 pavilions.

The small-scale experiment shows that horseshoe-shaped deposition patterns developed on the leeside of the containers. These depositions follow daily wind changes and leave deposits corresponding to the residual wind direction over the whole measuring period. Similar patterns are found around the beach pavilion, but, due to anthropogenic influences like bulldozing and beach shaping, longer term patterns in the direct vicinity of the pavilion and the dunes are hard to discern.

Evaluation of the longer term dataset reveals large variations in dune height and volume in the neighborhood of beach pavilions. Dune height/volume increases vary between 1-8 m in dune height and vary between 0-200 m3 in dune volume after 15 years along 2.7 km of coast. An

Figure 1: Overview of field sites locations represented in the Scanex, Coastscan and Airborne lidar datasets. Figure 1(A-C) shows the location of the Scanex2020 experiment with the location of the Coastscan experiement shown in Figure 1(D-F). Figure 1(G-H) shows the beach-dune system at Noordwijk (Figure 1I).

autocorrelation analysis shows that the alongshore variability length scale in dune volume of urbanized dunes can be 10 times smaller than for natural dunes. For about half the beach pavilions, variations in dune height and volume are significantly correlated to the location of the beach pavilion. Here the growth behind the buildings is lower than in the surrounding area which might have consequences for long-term resilience against future climate changes.

# Available data

Three different data set from various laser scan systems has been collected . The datasets consist of (1) the three-month Scanex2020 (Poppema et al., 2023) field campaign (order 100 meters) to study the influence of a simplified building on aeolian sand depositions (figure 1A-C), (2) the 2-year CoastScan (Kuschnerus et al., 2023 and Vos et al., 2023) field study (order 500 meters) to determine the influence of a single beach pavilion in front of a dune on longer term dune development (figure 1D-F), and (3) a 15-year long dataset of yearly airborne lidar scans to study the alongshore dune variability on the Noordwijk urbanized beach-dune system due to multiple beach pavilions along a 2.7-kilometer stretch of coast (1G-H).

The observations on the Scanex2020 field campaign were obtained with a mobile Terrestrial Laser Scanner (TLS, see table 1 for instrument specifications). For the CoastScan dataset observations have been obtained with a Permanent Laser Scanner (PLS) station on top of a hotel and finally airborne observations are obtained with various airborne lidar instruments (specifications not known) and obtained from governmental repositories (RWS, 2023)

The TLS and PLS scans have been combined in 1x1 meter grids by gridding all points in 1x1 meter grids without interpolation. After manual cleaning all points within the square meter are averaged in the z-direction. In total 31 grids were created for the 2019-2021 time period.

Table 1: Laser scan specifications and overview of measurements in measurement sets.

Figure 2: Grid filled PLS/TLS scan data indicated by color. PLS points are indicated with dark blue and other colors indicate individual TLS scans around BP6 (see Figure 2D-E). The grid is in local coordinates of the PLS (with the center point of the axis at the base of the laser scanner) with negative X values directed towards the sea and Y coordinates alongshore the coast. The beach pavilion is located X=-180m and Y=0.

An example of the merge can be seen in Figure 2 which shows a grid of the beach-dune systems around Beach pavilion 6 (see Figure 1D) with either TLS or PLS sources indicated by color. The figure shows a grid in local coordinates (of the PLS) with the PLS measurements in blue and TLS indicated in green and red.

The laser scanner on top of the hotel was georeferenced with twelve 5-centimeter reflectors on both the hotel and objects around the hotel spread over a 180 degree window in the field of view. Coordinates of the reflectors were obtained with RTK-GNSS (real-time kinematic global navigation satellite system measurements). Due to a relocation of the laserscanner on 2 december 2020 a second calibration was performed after the shift. Two global transformation matrixs from local point coordinates to the Dutch reference system (RD-NAP, EPSG 28992, [Bruine et al., 2005]) are provided with the datasets to transform the data from local to global coordinates.

The ALS data has been obtained from RWS. Data from data block (24HZ1, 30EN2 and 30FN1) has been combined and the coast of Noordwijk has been extracted. Data before 2012 was provided as 5x5 meter geotiff-data and has been converted to X-Y-Z coordinates. A total of 16 data sets are available for the time period 2008-2023. All point clouds of the ALS data are in the Dutch RD-NAP coordinate system in meters.

Finally the locations of the dune toe/top/heel and the location of the beach pavilions are provided as auxiliary data.

# Data Format

TLS/PLS/ALS data

All laser scan data are provided in X-Y-Z coordinates in meters and stored as ascii files.

Grid data from the TLS and PLS are provided in local X-Y-Z Cartesian coordinates with the PLS sensor as origin (X=0.0, Y=0.0) and the height referenced to RD-NAP (by adding the height of the laser scanner ( 55.79 meter) to the Z-coordinates).

ALS data is provided as X-Y-Z coordinates  in the Dutch RD-NAP coordinate system.

For transformation to RD-NAP coordinates grid data should be converted to local PLS coordinates (with the PLS sensor as origin for all axis (X-Y-Z). The data can be transformed to RD-NAP using the provided global transformation matrixes (‘Global_transformation_matrix_NW_before_2dec2020.dat’ and ‘Global_transformation_matrix_NW_after_2dec2020.dat’).

Units:

Meters

File format:

All data files are stored as X-Y-Z ascii files

Data attributes in the TLS/PLS point cloud data:

| X Y Z | 3D coordinates of the TLS/PLS measurements are in the local coordinate system (in meters). Height data is referenced to the Dutch national coordinates (RDNAP) with elevations relative to the mean sea level. |
| --- | --- |

Data attributes in the ALS point cloud data:

| X Y Z | 3D coordinates of the TLS/PLS measurement are in the Dutch national coordinates system (RDNAP) with elevations relative to the mean sea level. |
| --- | --- |

Additional files:

| Global alignment matrixes (Global_transformation_matrix_NW_before_2dec2020 and Global_transformation_matrix_NW_after_2dec2020) | Contains a 4 x 4 rigid transformation matrix to georeferenced points in RD-NAP, EPSG 28992, [Bruine et al., 2005]. |
| --- | --- |

| Dune properties Noordwijk_Dunetoe.xy Noordwijk_Dunetop.xy Noordwijk_Duneheel.xy | Contains the X-Y locations of the dune toe/top/heel in local PLS coordinates. |
| --- | --- |

| Beach pavilion locations Noordwijk_locations_beach pavilions.y | Contains the Y-locations of the beach pavilions along the dune toe. |
| --- | --- |

File naming convention:

PLS and TLS data are stored per time-epoch with the format YYMMDD-hhmmss with YY specifying the year, MM, the month, DD, the day of the month, and the time of the measurement in hours (hh), minutes (mm), and seconds (ss). ALS data is stored per year.

# References

- Bruijne, A. de, Buren,J. van, Kösters, A. & Marel, H. van der, Geodetic reference frames in the Netherlands; Definition and specification of ETRS89, RD and NAP, and their mutual relationships, Netherlands Geodetic Commission 43, Delft, 132, (2005) , ISBN 90 6132291 X.

- Kuschnerus, M., Lingenbergh, R.C., Vos, S.E & Hanssen, R. (2023) Statistically Assessing Vertical Change on a Sandy Beach from Permanent Laser Scanning Time Series, IRPS, International Archives of Photogrammetry, Remote Sensing and Spatial Information Sciences, (Accepted, not yet published,).

- Poppema, D.W., Wijnberg, K.M., Mulder, J.P.M, Vos, S.E. & Hulscher S.J.M.H. (2021) The effect of building geometry on the size of aeolian deposition patterns: Scale model experiments at the beach, Coastal Engineering 168, pp. 103866, www.doi.org/10.1016/j.coastaleng.2021.103866.

- RWS (Rijkswaterstaat, Ministry of Public Works), data obtainable via download.rijkswaterstaat.nl (Square 24HZ1, 30EN2 and 30FN1), last visited 15-11-2023.

- Vos, S., Lindenbergh, R., de Vries, S., (2017). CoastScan: Continuous Monitoring of Coastal Change using Terrestrial Laser Scanning, Proceedings of Coastal Dynamics 2017, 233, pp.1518-1528, https://coastaldynamics2017.dk/onewebmedia/233_Vos_Sander.pdf.

- Vos, S.E., Kuschnerus, M., Lindenbergh, R.C, and Vries, S. de (2023): 4D spatio-temporal laser scan dataset of the beach-dune system in Noordwijk, NL. Version 2. 4TU.ResearchData. dataset. https://doi.org/10.4121/1aac46fb-7900-4d4c-a099-d2ce354811d2.v2

- Vos, S.E., IJzendoorn, C. van, Lindenbergh, R.C and Wulf, Alain de (2024) Asynchronous dune development on on a Dutch urbanized beach due to buildings and other anthropogenic influences. Geomorphology. DOI.