# Groyne Measurements

## Introduction

In this exercise the profile of a groyne will be measured. The same measurements were done in 2002 so if large changes have occurred they can be discovered. In 2002 a fixed point was chosen from which the groyne was mapped. This point was chosen in such a way that future measurements can take place from the same starting point leading to a series of measurements of this groyne spread out over several years. In such a way a good indication can be found about the stability of the groyne itself and the armour blocks in particular. To get a first impression about the change of the profile, the profiles are measured and then are plotted in order to be investigated visually. To get a better insight in the change of the amount of material a calculation of the total volume of the groyne is made.

## Location of the starting point

At the end of the breakwater a fixed point was located that was easy to recognize and would not move or disappear in future. As can been seen the right side of the breakwater and outlet channel, seen from the shoreline on, was chosen to fix this point (Figure 1-1).

Figure 1-1 Groyne drawing

Figure 1-2 Starting point

As a starting point for the measurements the bended piece of the breakwater was ignored. Notice that at the last horizontal concrete plate (the one before the slope) a little corner was missing ( and ). This corner was taken as the reference point of the line stipulated along the breakwater (x = 0, x-axis along the breakwater).

The starting point of the line is set 1.5 m perpendicular to this corner (Figure 1-3). Every 5 meters a point was marked, and every 10 meters a cross-profile was measure perpendicular to this line, starting at L = 5 m, ending at L = 35 m.

Figure 1-3 Detailed drawing starting point

## Profile measurements of a groyne

The profile of the groyne was measured using the same reference points as last year. With a distance of 10 meter between each other 4 profile measurements were carried out using a hemisphere of 0.75 m attached to a rod. This hemisphere is used to level out the influence of an individual block on the measurement. The size of this hemisphere should be approximately ½* d50. In 2002 the same groyne was measured, using a hemisphere with a dimension of 0.25 m, and these measurements are used for comparison. The compare the different profiles the point at L= 5.0 m, Y = 0 is taken as the zero point in Z-direction and every point is calculated in reference to that. This leads to the profiles in Figure 1-4. All the data are collected in Error: Reference source not found. These profiles show some difference between the measurements in 2002 and 2003.

Figure 1-4 Profiles at different cross-sections

A part of the differences between the measurements can be attributed to the use of different hemispheres. As has been noted before two hemispheres were used. In 2002 only the small hemisphere, with a diameter of 0.25 meter, was used. In 2003 most of all the cross sections were measured with the large hemisphere, diameter 0.75 meters, but for a reference also measurements using the small hemisphere and no sphere at all were carried out. This was done for one profile, the profile at L = 15 meter. In Figure 1-5 the 3 different profiles from these 3 measurements are shown.

Figure 1-5 Profiles with different hemispheres

From this comparison it becomes apparent that the use of a different hemisphere has some effect on the profile that is measured. The large hemisphere flattens out small peaks in the profile where the small hemisphere, or just the rod without the use of a hemisphere, fits in the crevice between two stones. If the measurements for cross-section at L = 15 meter, done in 2002 with the small hemisphere are compared with the measurements from 2003 also made with the small hemisphere a better indication for the actual change in profile can be made visible. Also in this case differences occur between the measurements. Apparently material of the groyne has been moved throughout the year. Important to know in this case is whether the total amount of material of the groyne remains the same, is there change of volume?

Figure 1-6 Profile at L=15 m, small hemisphere

The change of volume can be computed from the measurements. The cross-sections are known; the distance between the cross-sections is known, so also a rough estimate of the volume can be computed. Difficult in this case is the lower boundary, almost every cross-section measurement ends somewhere along the waterline. A fixed lower boundary is not to be found, so some value will be chosen. From looking at the data it becomes clear that no data exceeds more than 2 meter below the before chosen zero point. So 2 meter is chosen as the lower boundary. In Figure 1-7 this is shown for the profile at L = 5 meter. The surface of this profile is calculated between the lines Y = 0m, the blue line along the Y-axes, and Y = 9.5 m, the blue line at Y (width) is 9.5 meter. Above and below this area is bounded by Z= 0 m and the profile line, also given in blue.

Figure 1-7 Profile volume calculation

In this particular case the surface is 15.9 m2. Similar calculations were carried out for every cross-section for both the 2002 and the 2003 data. The results are shown in Table 1-1.

|  |  | Profile |  |  |
| --- | --- | --- | --- | --- |
| year | L = 5 m | L = 15 m | L = 25 m | L = 35 m |
| 2002 | 15.9 | 25.9 | 27.4 | 30.3 |
| 2003 | 16.6 | 27.4 | 27.6 | 32.6 |

Table 1-1 Surface areas for profiles

Now the total volume of the groyne can be calculated. This leads for the data from 2002 to a total volume of 764 m3 and for the data from 2003 to a volume of 796 m3. The difference is rather small, just a few percent, and can be mainly attributed to the use of the different hemisphere. Also the volume in 2003 is a bit larger than it was in 2002. This means that the amount of rock in the groyne has increased while no suppletions have taken place. This can be explained by the fact that the measurements took place above the water level. Some material can be moved from below the water level and placed above, leading to an increase in the amount of rock. Although these mechanisms take place the differences are mainly due to the use of a different hemisphere.

## Conclusion

Part of the differences between the measurements of this year and those of last year can be accounted to the use of a different hemisphere, but also some displacement of material has taken place.