// -*-c++-*-

/** \page setup How to set up your application

Global set-up hints and concepts on DUECA project organisation

\section modules Blocks and modules

DUECA facilitates the implementation of a data-flow architecture. Many

real-time simulations have a data-flow nature. Consider the simple

example in the accompanying diagram:

\image html images/dataflowblock.png "Block diagram for a simple simulation"

The motions of the real-aircraft can be characterised as changes in

continuous physical variables, and a block diagram is a common format

for representing the structure of a model of the aircraft. In a block

diagram, the arrows represent signals, also typically expressed as

continuous functions of time.  In a digital simulation, all blocks

have to be calculated with an update rate that is high enough to make

the difference between the real "analogue" system and the digital

implementation small enough to be unnoticeable.

In DUSIME, each block in a block diagram is normally represented by a

"module". Two types of DUSIME modules are possible:

<ul>

<li> A %SimulationModule. For a piece of simulation that is

model-specific, and that does not do any (direct) communication with

hardware, you should implement a %SimulationModule. You do this by creating

a class that is derived from the dueca::SimulationModule class (Note for

novice C++ programmers: \ref deriveclass "Derived Classes").

<li> A %HardwareModule. This type of module is used to implement a

piece of simulation that directly interfaces with the hardware of a

simulator, for example the interfacing code for control loading

hardware. A %HardwareModule is implemented by creating a class that is

derived from the dueca::HardwareModule class. Note that you only need

a %HardwareModule when you have to be careful with the hardware or

with the user attached to that hardware. For interaction with a

harmless device that does not require a specific set-up sequence

(joysticks, displays), you can also use a %SimulationModule or a plain

DUECA %Module.

</ul>

The primary difference between a %SimulationModule and a

%HardwareModule is that the latter receives commands from DUSIME that

tell it when and how to control the hardware, for example commands to

drive the hardware to a neutral position, perform a test on the

hardware IO, etc. A %HardwareModule also uses two main operation

modes, one is implemented as a safe fallback, in which the only task

is servicing the hardware, and the other for participation with the

simulation, in which the module also communicates with the rest of the

simulation.

Control loading hardware is controlled by such a HardwareModule. Upon

the simulaiton start, it will run through a sequence where it connects

to the hardware, activates the hardware, and then brings it slowly to

a neutral position. Only when that is ready the other modules in a

simulation are started and communication starts.

The main difference between a SimulationModule and a plain DUECA

%Module is that a SimulationModule distinguishes extra states useful

in simulations, namely keeping the simulation frozen, advancing and

replaying. A dueca::Module only runs or not. In this example, the

display may be a normal Module; the instrumentation has to be re-drawn

anyhow, with either the data from the frozen simulation or the data

from the moving simulation. However, if you want to show an extra

indicator telling a pilot to manually set the throttle to a starting

position while the simulation is frozen, you would need a

SimulationModule.

\section channels Data channels

In the block diagram given in the previous section, the data

connections between blocks are represented by lines with arrow ends

drawn between the blocks. The data that is "sent" over these lines

consists of scalar variables or vectors. In DUECA these data

connections are implemented by "channels", and the type of data that

can be sent is flexible. A DUECA diagram of the block diagram

given above is:

\image html images/dataflowdueca.png "Dueca data flow diagram for a simple simulation"

Each square block is a module, and each oval is a channel. Modules can

write to channels and read from channels. A module "lives" within one

DUECA node (a computer with DUECA running on it), and a channel can

(but does not necessarily have to) "flow" between different nodes,

i.e., it can be distributed between different DUECA nodes (something

which is not shown on the diagram, because we want to emphasize the

logical structure of the simulation).

Each channel can send a specific data type, but any kind of data can

be sent across, strings, floating point values, integers, vectors or

lists of these basic data types. The objects that can be sent through channels are called DUECA Communication Objects (DCO). In a DUECA project you will find these in the comm-objects (for communication objects) folders, and they have a filename with a '.dco' suffix. With a code generator, the DCO files are converted to C++ classes that can be sent through channels, here are some examples:

<ul>

<li> A data type for the primary controls (pitch, roll and yaw inputs

with stick or yoke and rudder), let's call it PrimaryControls, with

three data elements of type double:

\code

(Type double)

;; Aircraft primary control inputs.

(Stream PrimaryControls

;; stick or yoke angle [rad]

(double stick_pitch (Default 0.0))

;; lateral stick or wheel angle [rad]

(double stick_roll (Default 0.0))

;; rudder excursion [m]

(double rudder_yaw (Default 0.0))

)

\endcode

For more details see \ref codegenerator

The type of data needs to be declared in the file's header, standard C++

types (double, float, etc.) are available immediately.

The following is a segment of what comes out of the code generator:

\code

class PrimaryControls

{

public:

double stick_pitch;

double stick_roll;

double rudder_yaw;

// constructor, destructor and further members

};

\endcode

</li>

<li> A data type for the output of an aircraft model. let's call it

AircraftOutput, with an array of 20 elements of type double:

\code

class AircraftOutput

{

public:

dueca::fixvector<double,20> Y;

// constructor, destructor and further members

};

\endcode

You can see that this object uses a dueca::fixvector, a fixed-length

vector. It is best to make the objects that you send over a channel

have a fixed size, so memory allocation and de-allocation can be done

efficiently. Variable-length vectors, such as provided in the stl, can

be used, but incur an additional memory allocation and de-allocation

overhead which is often not acceptable in a real-time program.

</li>

<li> A data type for the input from a keyboard:

\code

class KeyPress

{

uint8_t key;

// constructor, destructor and further members

};

\endcode

</li> </ul> Note that there are many more (and possibly much more

complicated) examples of data that you may wish to send in a

simulation. Note also that you should not code these data classes by

hand; there is a code generator supplied with DUECA that generates

these classes from a ".dco" (DUECA Communication Object)

specification. It adds the code needed to pack and unpack the DCO

objects, and optionally provides extra code for e.g., logging such an

object.

\section channeltypes Channel types

As was mentioned above, a channel handles a specific type of data, but

channels can be set up for (almost) any type of data. Before DUECA

2.0, several channel types existed. In DUECA 2.0 these were unified

into a single, very flexible, channel. However, for most common use

the good old channels are still sufficient. The programming interface

for these types is still available, and since they are a bit simpler

to understand and explain, the explanation starts with these two

types:

<ul>

<li> StreamChannel. A stream channel treats its data as a continuous

update of some output or state value. This approximates a continuous,

analogue value in the real world. The stream data should be updated

regularly, and at each time it has a single, specific value. When

stream data is updated, it is updated for a logical time <i>span</i>,

indicating the period for which the data is considered valid.

<li> EventChannel. An event channel handles "event" data. An event is

an occurrence that is unique in time, has no "duration", and it has

some data associated with it. Event data is associated with a single

point in time, and in a specific channel, multiple events may be

associated with the same time. There may of course also be times that

there are no events written in the channel.

</ul>

Typically, the PrimaryControls and AircraftOutput data mentioned in

the \ref channels "previous section" would be implemented as stream

data, while the KeyPress data would be sent as event data.

\section highleveldesing Going about a design

The advised way of starting a design of a simulation for DUECA would

be to set up a data flow model for the simulated system. Identify what

parts of the simulated system you want to implement as an individual

block, and create a SimulationModule (or HardwareModule) class for

those parts. Carefully consider what data is transmitted between those

modules, and whether this is better represented as "continuous" stream

data or as discrete events.

*/