| Authors | Mario Luis Miranda Montenegro Shungudzemwoyo P. Garaba |
| --- | --- |
| Title | Raw absorbance and fluorescence measurements of water sample during Nodularia spumigena bloom event on Lake Bante in Wilhelmshaven, Germany |
| Keywords | absorbance, hyperspectral, flourescence, Lake Bante, Germany, BanterSee |
| Description | Water sampling was conducted from the shoreline of Lake Bante in Wilhelmshaven, Germany. A very dense Nodularia spumigena bloom was taking place on the day of sampling 16 August 2021. Samples were kept in the dark and stored at 4 °C until further analyses were conducted. First the samples were pre-filtered through Whatman GF/F filters with a 0.7 µm mesh. Further filtration was done using Whatman Nuclepore Track-Etch 0.2 µm pore size membranes within 72 hours after field sampling. Sample filtrate was diluted using deionized water at a ratio 1:4 to mitigate inner filter effect and detector saturation. Excitation-emission matrices (EEMs) and absorbance measurements were done using a Horiba Aqualog benchtop fluorometer. Sample filtrate was placed in a 1 cm quartz cuvette that had been pre-rinsed 3 times before each measurement. |
| Instrument | Horiba Aqualog benchtop fluorometer, 1 cm quartz cuvette |
| Geo-location | Lake Bante, Wilhelmshaven, Germany |
| License | Creative Commons Attribution 4.0 |

| Station | Date/Time (UTC) | Latitude (° N, decimal) | Longitude (° E, decimal) | Filename |
| --- | --- | --- | --- | --- |
| 1 | 2021-08-16T11:00:00 | 53.5093 | 8.114 | Banter See S1 |

ABS Folder

| Description | Absorbance measurements exported into 11 columns. To use this data to perform PARAFAC Analysis columns 1 and 10 must be used for corrections. |
| --- | --- |

Blank Folder

| Description | Blank fluorescence emission was recorded accordingly with the EEMS (240-600 nm). Blank fluorescence emission was recorded every 2 nm. Blanks were excited with wavelengths between 213 to 622 nm Blank excitation wavelengths are applied in a pre-set stepping (here 1.677 nm). The corrections for PARAFAC analysis should be performed before dataset assembling. |
| --- | --- |

EMMS Folder

| Description | Samples fluorescence was recorded from 213 to 622 nm. Fluorescence is recorded in a pre-set stepping (here 1.677 nm). The corrections for PARAFAC analysis should be performed before dataset assembling. |
| --- | --- |