Files content: Съдържание на файловете:

- There are separate files for every cycle of measurement:

а.  Files with extension *.GPS contain processed data gathered by GPS;

б.  Files with extension *.DEP contain data from the echo sounder;

в.  Files with extension *.COR contain geodetic coordinates, equivalent to those from the GPS;

г.  Files with extension *.TXT contain coordinates and waterdepths for modeling the seabed with software like AutoCAD, Arc/View, Surfer, etc.

- GPS date and echosounder data are synchronized in time according following procedure:

# А. For the first measurement cycle on 8.10.2002:

-    Start time of GPS:    8:55:00

- The echosounder at the same moment shows 0:03:36;

- Example: for a point, measured by GPS at 9:11:55 time difference t is:

9:11:55-8:55:00 = 0:16:55= t. Echosounder’s time thus is: 0:03:36 + 0:16:55 =0:20:31. At this moment the echosounder has measured (see file *.dep) 8.77 m в т.75 .

Б. For the second measurement cycle on 8.10.2002::

-    Start time of GPS:    11:22:50

- The echosounder at the same moment shows 0:03:20;

- Пример: Exampleр: for a point, measured by GPS at  11:26: 55 time difference t is:

11:26:05-11:22:50 = 0:03:15= t. Echosounder’s time thus is: 0:03:20 + 0:03:15 = 0:06:35. At this moment the echosounder has measured (see file *.dep) 0.74 m.

3. There is a drawing “Drawing1.dwg” for Autocad. (I do not see anything in that file!!, B.S)

(This is a translation of the Bulgarian file I received from Sofia, Boyan)