# A. Excel tool pictures

# B Underlying data

### B.1 Rainfall time series

- Return period of 2 years is considered

- The base rainfall for calculation of the pipe dimensions is taken as “bui 07” – one of the commonly used design showers in the Netherlands (RIONED, 2019). The total volume os this rainfall is 19.8 mm and the peak is 39.6 mm/hr

- The challenge in the game was to upgrade or lay a new sewer system for the rainfall (RIONED, 2020) that incorporates climate change (2030 rainfall from RIONED is taken as the new design rainfall from 2024 untill 2050). The total volume of the rainfall is 39.4 mm and the peak (at 5 hour) is 78.9 mm/hr.

- For the rainfall intensity calculations, the peak is used using rational method

| Base case |  | New design rainfall |  |
| --- | --- | --- | --- |
| Timestep (hour) | Volume (mm/hr) | Timestep (hour) | Volume (mm/hr) |
| 0.08 | 7.2 | 0.08 | 1.0 |
| 0.17 | 14.4 | 0.17 | 1.0 |
| 0.25 | 25.2 | 0.25 | 1.0 |
| 0.33 | 39.6 | 0.33 | 1.1 |
| 0.42 | 39.6 | 0.42 | 1.1 |
| 0.50 | 32.4 | 0.50 | 1.1 |
| 0.58 | 25.2 | 0.58 | 1.1 |
| 0.67 | 18.0 | 0.67 | 1.1 |
| 0.75 | 14.4 | 0.75 | 1.1 |
| 0.83 | 10.8 | 0.83 | 1.2 |
| 0.92 | 7.2 | 0.92 | 1.2 |
| 1.00 | 3.6 | 1.00 | 1.2 |
|  |  | 1.08 | 1.2 |
|  |  | 1.17 | 1.3 |
|  |  | 1.25 | 1.3 |
|  |  | 1.33 | 1.3 |
|  |  | 1.42 | 1.3 |
|  |  | 1.50 | 1.4 |
|  |  | 1.58 | 1.4 |
|  |  | 1.67 | 1.4 |
|  |  | 1.75 | 1.4 |
|  |  | 1.83 | 1.5 |
|  |  | 1.92 | 1.5 |
|  |  | 2.00 | 1.5 |
|  |  | 2.08 | 1.6 |
|  |  | 2.17 | 1.6 |
|  |  | 2.25 | 1.6 |
|  |  | 2.33 | 1.7 |
|  |  | 2.42 | 1.7 |
|  |  | 2.50 | 1.8 |
|  |  | 2.58 | 1.8 |
|  |  | 2.67 | 1.9 |
|  |  | 2.75 | 1.9 |
|  |  | 2.83 | 2.0 |
|  |  | 2.92 | 2.0 |
|  |  | 3.00 | 2.1 |
|  |  | 3.08 | 2.2 |
|  |  | 3.17 | 2.2 |
|  |  | 3.25 | 2.3 |
|  |  | 3.33 | 2.4 |
|  |  | 3.42 | 2.5 |
|  |  | 3.50 | 2.6 |
|  |  | 3.58 | 2.7 |
|  |  | 3.67 | 2.8 |
|  |  | 3.75 | 2.9 |
|  |  | 3.83 | 3.0 |
|  |  | 3.92 | 3.2 |
|  |  | 4.00 | 3.3 |
|  |  | 4.08 | 3.5 |
|  |  | 4.17 | 3.7 |
|  |  | 4.25 | 4.0 |
|  |  | 4.33 | 4.3 |
|  |  | 4.42 | 4.6 |
|  |  | 4.50 | 5.1 |
|  |  | 4.58 | 5.7 |
|  |  | 4.67 | 6.5 |
|  |  | 4.75 | 7.9 |
|  |  | 4.83 | 10.6 |
|  |  | 4.92 | 18.2 |
|  |  | 5.00 | 78.9 |
|  |  | 5.08 | 78.9 |
|  |  | 5.17 | 18.2 |
|  |  | 5.25 | 10.6 |
|  |  | 5.33 | 7.9 |
|  |  | 5.42 | 6.5 |
|  |  | 5.50 | 5.7 |
|  |  | 5.58 | 5.1 |
|  |  | 5.67 | 4.6 |
|  |  | 5.75 | 4.3 |
|  |  | 5.83 | 4.0 |
|  |  | 5.92 | 3.7 |
|  |  | 6.00 | 3.5 |
|  |  | 6.08 | 3.3 |
|  |  | 6.17 | 3.2 |
|  |  | 6.25 | 3.0 |
|  |  | 6.33 | 2.9 |
|  |  | 6.42 | 2.8 |
|  |  | 6.50 | 2.7 |
|  |  | 6.58 | 2.6 |
|  |  | 6.67 | 2.5 |
|  |  | 6.75 | 2.4 |
|  |  | 6.83 | 2.3 |
|  |  | 6.92 | 2.2 |
|  |  | 7.00 | 2.2 |
|  |  | 7.08 | 2.1 |
|  |  | 7.17 | 2.0 |
|  |  | 7.25 | 2.0 |
|  |  | 7.33 | 1.9 |
|  |  | 7.42 | 1.9 |
|  |  | 7.50 | 1.8 |
|  |  | 7.58 | 1.8 |
|  |  | 7.67 | 1.7 |
|  |  | 7.75 | 1.7 |
|  |  | 7.83 | 1.6 |
|  |  | 7.92 | 1.6 |
|  |  | 8.00 | 1.6 |
|  |  | 8.08 | 1.5 |
|  |  | 8.17 | 1.5 |
|  |  | 8.25 | 1.5 |
|  |  | 8.33 | 1.4 |
|  |  | 8.42 | 1.4 |
|  |  | 8.50 | 1.4 |
|  |  | 8.58 | 1.4 |
|  |  | 8.67 | 1.3 |
|  |  | 8.75 | 1.3 |
|  |  | 8.83 | 1.3 |
|  |  | 8.92 | 1.3 |
|  |  | 9.00 | 1.2 |
|  |  | 9.08 | 1.2 |
|  |  | 9.17 | 1.2 |
|  |  | 9.25 | 1.2 |
|  |  | 9.33 | 1.1 |
|  |  | 9.42 | 1.1 |
|  |  | 9.50 | 1.1 |
|  |  | 9.58 | 1.1 |
|  |  | 9.67 | 1.1 |
|  |  | 9.75 | 1.1 |
|  |  | 9.83 | 1.0 |
|  |  | 9.92 | 1.0 |
|  |  | 10.00 | 1.0 |

## B.2.  Hydraulics

### Sub-catchment details

Run-off coefficient values per infrastructure type

Depending on the perviousness of the area where rain falls, it either largely infiloitrates into the ground or runs off into the sewer system. For instance, rain that falls on green spaces is assumed to mostly infilitrate into the ground while rain that falls on roofs, streets and sidewalks predominantly enters the sewer system. Therefore, the percentage of strormwater that enters the sewer system depends upon the level of imperviousness of the receiving area, often expressed as a runoff coefficient between 0 and 1 (0 = 100% pervious, 1 = 100% impervious). Values of the runoff coefficients can be found in the table below. The values of typical urban infrastructure have ben taken from (Butler & Davies, 2011). The C values of SUDS such as standard sedum roof, intensive green roof and permeable pavements have been taken from pilot projects described in Pötz & Bleuzé (2012) and the others have been assumed. We make further assumptions to translate the capacity of SUDS that store, rather than infilitrate water, by assuming that they drain/disconnect a certain percentage of roof or garden area where 1 unit of roof area is taken as 52m2 and 1 unit of garden area is taken as 36m2 (typical values for a Dutch standalone house).

| Area description | Typical range of C values (Butler & Davies, 2011) | Selected value | Area connected/drained |
| --- | --- | --- | --- |
| Typical urban infrastructure |  |  |  |
| Asphalt and concrete paving | 0.70-0.95 | 0.83 |  |
| Lawns | 0.05-0.35 | 0.20 |  |
| Parks and gardens | 0.05-0.30 | 0.18 |  |
| Roofs | 0.75-0.95 | 0.85 |  |
| Residential | 0.30-0.70 | 0.50 |  |
| SUDS |  |  |  |
| Standard sedum roof |  | 0.45 |  |
| Intensive green roof |  | 0.25 |  |
| Rainwater fences |  | 0.4 | 1/4th of a roof |
| Rain barrels |  | 0.4 | 1/4th of a roof |
| Rainwater storage tank |  | 0.3 | 1 garden and 1 roof |
| Permeable pavement |  | 0.4 |  |
| Rain garden |  | 0.35 |  |
| Green facade |  | 0.45 |  |
| Vertical garden |  | 0.4 |  |

Base case setup of sub-catchments and division of areas

The overall case study area was divided into 11 sub-catchments. The table below shows the details of each sub-catchment area. Using aereal images from Google maps, each sub-catchment area was roughly divided into more specific areas for different infrastructure types across public and private space.

|  |  | Public area |  | Private area |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sub-catchment (going clockwise) | Area (ha) | Area paving (m2) | Area Parks + gardens (m2) | Area lawns (m2) | Area roofs (m2) | Area outside space (m2) | Area wall space (m2) | Base case sum Ci x Ai |
| S1 | 0.19 | 372 | 186 | 93 | 372 | 669.6 | 167.4 | 0.11 |
| S2 | 0.24 | 242 | 363 | 363 | 605 | 677.6 | 169.4 | 0.13 |
| S3 | 0.26 | 255 | 127.5 | 255 | 1020 | 714 | 178.5 | 0.16 |
| S4 | 0.25 | 375 | 250 | 375 | 625 | 700 | 175 | 0.14 |
| S5 | 0.17 | 255 | 85 | 170 | 680 | 340 | 85 | 0.10 |
| S6 | 0.14 | 280 | 0 | 0 | 560 | 448 | 112 | 0.10 |
| S7 | 0.14 | 140 | 0 | 70 | 840 | 280 | 70 | 0.10 |
| S8 | 0.23 | 231 | 0 | 0 | 1155 | 739.2 | 184.8 | 0.16 |
| S9 | 0.25 | 372 | 124 | 124 | 992 | 595.2 | 148.8 | 0.16 |
| S10 | 0.24 | 244 | 122 | 122 | 976 | 780.8 | 195.2 | 0.16 |
| S11 | 0.21 | 206 | 103 | 103 | 1236 | 329.6 | 82.4 | 0.15 |
| Total area | 2.31 | 2972 | 1361 | 1675 | 9061 | 6274 | 1568.5 |  |

| Sub-catchment | Area (ha) | Fixed area |  |  | Changeable area |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Area roofs (m2) | Area roofs |  | Area residential (m2) | Area outside space |  |  |  |  | Area wall space |  |  |
|  |  | Area public paving (m2) | Area lawns (m2) | Area Parks + gardens (m2) |  | Area standard green roofs (m2) | Area intensive green roofs (m2) |  | Area rainwater fences (m2) | Area rain barrels (m2) | Area storage tanks (m2) | Area permeable pavement (m2) | Area rain garden (m2) | Area green façade (m2) | Area vertical garden (m2) | New sum Ci x Ai (ha) |
| S1 | 0.19 | 372 | 93 | 186 | 279 | 93 | 0 | 671 | 48 | 0 | 0 | 67 | 0 | 50 | 0 | 0.10 |
| S2 | 0.24 | 242 | 363 | 363 | 454 | 151 | 0 | 679 | 49 | 0 | 0 | 68 | 0 | 51 | 0 | 0.12 |
| S3 | 0.26 | 255 | 255 | 128 | 765 | 255 | 0 | 716 | 52 | 0 | 0 | 71 | 0 | 54 | 0 | 0.15 |
| S4 | 0.25 | 375 | 375 | 250 | 469 | 156 | 0 | 702 | 51 | 0 | 0 | 70 | 0 | 53 | 0 | 0.13 |
| S5 | 0.17 | 255 | 170 | 85 | 510 | 170 | 0 | 341 | 25 | 0 | 0 | 34 | 0 | 26 | 0 | 0.10 |
| S6 | 0.14 | 280 | 0 | 0 | 420 | 140 | 0 | 449 | 32 | 0 | 0 | 45 | 0 | 34 | 0 | 0.09 |
| S7 | 0.14 | 140 | 70 | 0 | 630 | 210 | 0 | 281 | 20 | 0 | 0 | 28 | 0 | 21 | 0 | 0.09 |
| S8 | 0.23 | 231 | 0 | 0 | 866 | 289 | 0 | 741 | 53 | 0 | 0 | 74 | 0 | 55 | 0 | 0.15 |
| S9 | 0.25 | 372 | 124 | 124 | 744 | 248 | 0 | 597 | 43 | 0 | 0 | 60 | 0 | 45 | 0 | 0.15 |
| S10 | 0.24 | 244 | 122 | 122 | 732 | 244 | 0 | 783 | 56 | 0 | 0 | 78 | 0 | 59 | 0 | 0.15 |
| S11 | 0.21 | 206 | 103 | 103 | 927 | 309 | 0 | 331 | 24 | 0 | 0 | 33 | 0 | 25 | 0 | 0.13 |
| Total area | 2.31 |  |  |  | 6795.75 | 2265 | 0 | 6291.50 | 453 | 0 | 0 | 627 | 0 | 471 | 0 |  |

Implementation of SUDS at sub-catchment level

The implementation of SUDS at the sub-catchment level was done by changing the C value of the sub-catchment.  If more SUDS are implemented, the runoff coefficient of the area decreases, leading to less runoff water going to the sewers. Since we only focus on private household SUDS in the game, we considered public spaces such as pavings, parks and gardens and lawns as  fixed and  only outside and wall spaces of  houses were considered as available area for implementing SUDS. For instance, in the table below, the tarmac roof area would decrease if the players implement standard green roofs or intensive roofs. Similarly, the paved outside space and wall space area would decrease if residents implement SUDS in their front yards/backyards or walls respectively.

### Hydraulic and flooding calculations methodology

Laying down of pipes:

Some assumptions made for the pipes in the layout:

- A separated sewer system is assumed to be installed and the layout is made only for stormwater pipes

- Since Netherlands is a flat country, there is no natural surface elevation to allow for water to travel under the influence of gravity. Hence, the pipes are laid out with a slope so that the stormwater can be discharged into a receiving water body. A slop of 1:200 is taken for pipes at system ends and 1:500 for all other pipes in the system.

- A pipe clearance of 1m under the road is assumed

- All pipes are considered to be PVC pipes with size dimensions 0.2m, 0.25m, 0.3m, 0.35nm, 0.4m, 0.45m, 0.5m, 0.55m and 0.6m

- The friction factor (;see eq 3 below) for PVC pipes is taken as 0.045

Calculation of stormwater flows:

When it rains, rainwater runs off impervious areas and ends up in the sewer system. To determine the amount of stormwater received by the sewer system, the rational method is used. In this method, the peak discharge is related to rainfall intensity by the formula:

|  | Q=iCA | (1) |
| --- | --- | --- |

Where Q is the stormwater flow (m3), i is the rainfall intensity in m3s/ha, C is the surface runoff coefficient, and A is the catchment size (in ha). Table X and Y presents the expected stormwater flows in each sewer pipe based on calculations using the rational method for the existing sewer system upgrade layout and for new sewer system layout.

Once the stormwater flows are known, the sewer pipes can be dimensioned so as to accommodate the expected flow. The minimum diameter required to accommodate the total flow in a pipe segment can be calculated using eq. 2 where Q is the flow in the pipe segment u is the flow velocity (taken as 1.5 m/s), and D is the pipe diameter (in m).

|  | Q=u.πD24 | (2) |
| --- | --- | --- |

After rainfall events, sewer systems start filling up and work under pressurized flow. During these load conditions, it is critical to ensure that no flooding occurs in the system. In hydraulic terms, this means that the hydraulic gradient line (HGL) should remain below the ground level at all points in the sewer system. Under pressurized conditions, frictional head losses across a pipe segment can be calculated using the Darcy Weisbach equation:

|  | ∆h=Lu22Dg | (3) |
| --- | --- | --- |

Where  is the friction factor, L is the pipe length (m), u is the flow velocity (m/s), D is the pipe diameter (m), and g is the gravitational constant (= 9.81 ms2). On substituting D from u = 4 QπD2 in the Darcy Weisbach equation, we get:

|  | ∆h=L*16*Qtot2π22g* D5 | (4) |
| --- | --- | --- |

From equation 4, it is clear that selecting an appropriate diameter is critical as head loss across a pipe is proportional to 1D5. To check for flooding in the system, head losses across each pipe segment were calculated (starting from the water level in the receiving body assumed at -0.95m) using the Darcy Weisbach equation and for each pipe segment a check was made if the HGL at the downstream and upstream manhole crossed the ground level at that point.

In the Urban drain game, players were given the option of seelctig the diameter of the pipes in a specific layout and they could get feedback on the amount of flooding their choic leads to. The pipes were divided into four categories: upstream, middle, downstream and end pipes, as shown in the Tables below

### Hydraulic and flooding calculations existing sewer system upgrade

| Section | Catchment Area |  | Wet weather flow | Cummulative flow | Proposed Pipe |  |  |  |  |  |  |  |  | Piezometric level |  |  |  |  | Flooding? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conduit | Contributing subcatchment area | Overall cA |  |  | Ground Level |  | Invert level |  | Slope | Length | Diamater design req. | Pipe diameter (input by players) | Full pipe volume | Friction factor | Velocity | Head Loss | Head | Head |  |
|  |  | A | Qwwf=i.C.A | ∑Qwwf | Up | Down | Up | Down |  |  |  |  |  | l | Vt | Dh | Lower end | Upper end |  |
| [-] |  | ha | m3/s | [m3/s] | m | m | m | m | % | m | m | m | m3 | - | m/s | m | m AD | m AD | True/ false |
| C6 | S6 | 0.10 | 0.022 | 0.022 | -0.49 | 0 | -1.69 | -2.00 | 0.005 | 62 | 0.136 | 0.20 | 1.95 | 0.014 | 0.69 | 0.11 | 0.56 | 0.67 | TRUE |
| C5 | S5 | 0.10 | 0.023 | 0.045 | 0 | -0.53 | -2.00 | -2.13 | 0.002 | 67 | 0.195 | 0.30 | 4.74 | 0.013 | 0.63 | 0.06 | 0.50 | 0.56 | TRUE |
| C4 | S4 | 0.14 | 0.031 | 0.075 | -0.53 | -0.58 | -2.13 | -2.25 | 0.002 | 57 | 0.253 | 0.30 | 4.03 | 0.013 | 1.07 | 0.14 | 0.36 | 0.50 | TRUE |
| C3 | S3 | 0.16 | 0.035 | 0.110 | -0.58 | -0.61 | -2.25 | -2.37 | 0.002 | 62 | 0.306 | 0.30 | 4.38 | 0.013 | 1.56 | 0.33 | 0.03 | 0.36 | TRUE |
| C2 | S2 | 0.13 | 0.028 | 0.138 | -0.61 | 0.53 | -2.37 | -2.52 | 0.002 | 75 | 0.343 | 0.30 | 5.30 | 0.013 | 1.96 | 0.63 | -0.60 | 0.03 | TRUE |
| C1 | S1 | 0.11 | 0.024 | 0.162 | 0.53 | 0.14 | -2.52 | -2.63 | 0.002 | 53 | 0.371 | 0.35 | 5.10 | 0.013 | 1.69 | 0.28 | -0.88 | -0.60 | FALSE |
| C7 | S7 | 0.10 | 0.022 | 0.022 | -0.49 | -0.01 | -1.69 | -1.97 | 0.005 | 55 | 0.136 | 0.20 | 1.73 | 0.014 | 0.69 | 0.09 | -0.01 | 0.08 | TRUE |
| C8 | S8 | 0.10 | 0.022 | 0.044 | -0.01 | -0.47 | -1.97 | -2.09 | 0.002 | 62 | 0.193 | 0.30 | 4.38 | 0.013 | 0.62 | 0.05 | -0.07 | -0.01 | TRUE |
| C9 | S9 | 0.16 | 0.036 | 0.080 | -0.47 | -0.13 | -2.09 | -2.22 | 0.002 | 63 | 0.260 | 0.30 | 4.45 | 0.013 | 1.13 | 0.18 | -0.24 | -0.07 | TRUE |
| C10 | S10 | 0.16 | 0.034 | 0.114 | -0.13 | 0.91 | -2.22 | -2.36 | 0.002 | 70 | 0.311 | 0.30 | 4.95 | 0.013 | 1.62 | 0.40 | -0.65 | -0.24 | FALSE |
| C11 | S11 | 0.16 | 0.034 | 0.148 | 0.91 | 0.14 | -2.36 | -2.46 | 0.002 | 53 | 0.355 | 0.35 | 5.10 | 0.013 | 1.54 | 0.23 | -0.88 | -0.65 | FALSE |
| C12 | All | 1.42 |  | 0.311 | 0.15 | 0.06 | -2.46 | -2.49 | 0.002 | 14 | 0.514 | 0.45 | 2.23 | 0.012 | 1.95 | 0.07 | -0.95 | -0.88 |  |

### Hydraulic and flooding calculations new sewer system design

| Section | Catchment Area |  | Wet weather flow | Cummulative flow | Proposed Pipe |  |  |  |  |  |  |  |  | Piezometric level |  |  |  |  | Flooding? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conduit | Contributing subcatchment area | Overall C.A after implementing SUDS |  |  | Ground Level |  | Invert level |  | Slope | Length | Diamater design req. | Pipe diameter (input by players) | Full pipe volume | Friction factor | Velocity | Head Loss | Head | Head |  |
|  |  |  | Qwwf=i.C.A | ∑Qwwf | Up | Down | Up | Down |  |  |  |  |  | l | Vt | Dh | Lower end | Upper end |  |
| [-] |  |  | m3/s | [m3/s] | m | m | m | m | % | m | m | m | m3 | - | m/s | m | m AD | m AD | True/false |
| C1 | S6 | 0.09 | 0.020 | 0.020 | 0.05 | -0.5 | -1.15 | -1.46 | 0.005 | 62 | 0.131 | 0.20 | 1.95 | 0.014 | 0.64 | 0.09 | 0.22 | 0.31 | TRUE |
| C2 | S7 | 0.09 | 0.020 | 0.020 | 0.04 | -0.5 | -1.16 | -1.45 | 0.005 | 58 | 0.131 | 0.20 | 1.82 | 0.014 | 0.65 | 0.09 | 0.22 | 0.31 | TRUE |
| C3 | S5 | 0.10 | 0.021 | 0.062 | -0.5 | -0.45 | -1.45 | -1.55 | 0.002 | 48 | 0.229 | 0.30 | 3.39 | 0.013 | 0.88 | 0.08 | 0.14 | 0.22 | TRUE |
| C4 | S8 + S4 | 0.28 | 0.062 | 0.124 | -0.45 | -0.242 | -1.55 | -1.65 | 0.002 | 52 | 0.324 | 0.30 | 3.68 | 0.013 | 1.75 | 0.35 | -0.21 | 0.14 | TRUE |
| C5 | S3 + S9 | 0.29 | 0.064 | 0.188 | -0.242 | 0.07 | -1.65 | -1.79 | 0.002 | 70 | 0.400 | 0.40 | 8.80 | 0.012 | 1.50 | 0.25 | -0.45 | -0.21 | TRUE |
| C6 | S2+ S10 | 0.26 | 0.058 | 0.246 | 0.07 | 0.15 | -1.79 | -1.91 | 0.002 | 62 | 0.457 | 0.40 | 7.79 | 0.012 | 1.96 | 0.37 | -0.83 | -0.45 | FALSE |
| C7 | S11 | 0.13 | 0.029 | 0.029 | 0.98 | 0.15 | -0.22 | -0.50 | 0.005 | 56 | 0.158 | 0.20 | 1.76 | 0.014 | 0.93 | 0.17 | -0.83 | -0.65 | FALSE |
| C8 | S1 | 0.10 | 0.023 | 0.023 | 0.56 | 0.15 | -0.64 | -0.89 | 0.005 | 50 | 0.139 | 0.20 | 1.57 | 0.014 | 0.73 | 0.09 | -0.83 | -0.73 | FALSE |
| C9 | All |  |  | 0.298 | 0.15 | 0.06 | -1.91 | -1.94 | 0.002 | 14 | 0.503 | 0.40 | 1.76 | 0.012 | 2.38 | 0.12 | -0.95 | -0.83 | FALSE |

## B.3. Sewer performance assessment

Calculation of flood damage

To calculate flood damage, the output from the hydraulic tables are converted into damage costs. For instance, if there is flooding in a specific sub-catchment as shown in table below, then the flooding depth is used to look up the damage cost per house from the depth damage curve (see figure below) and that is multiplied with the number of houses in the sub-catchment. The damage cost is summed up over all the sub-catchments and is further multiplied into (2050 – 2024)/ 2 assuming that the design rainfall occurs every 2 years.

Flood damage per rain event = i=1No. of subcactchmentsFlood damage per housesi. No. of housesi

| Flooding? | Number of houses in the subcatchment | Average flooding depth | Flood damage per rain event | Flood damage for whole neighbourhood per until 2050 |
| --- | --- | --- | --- | --- |
| True/false | - | m | euros | euros |
| TRUE | 11 | 0.69 | 3290 |  |
| TRUE | 11 | 0.69 | 3338 |  |
| TRUE | 13 | 0.84 | 4864 |  |
| TRUE | 36 | 0.46 | 7536 |  |
| TRUE | 38 | 0.30 | 5140 |  |
| FALSE | 37 | 0.00 | 0 |  |
| FALSE | 16 | 0.00 | 0 |  |
| FALSE | 14 | 0.00 | 0 |  |
| FALSE | 175 | 0.00 | 0 |  |
|  |  |  | 24169 | 314197 |

Depth damage curve (y axis: flood damage in euros per house, x axis: flooding depth in cms) (Vogelzang, 2023)

Investment and maintenance costs

Assumptions:

- Investment cost is only considered for pipes that are replaced in the upgrade layout

- It is assumed that new pipes in the upgrade layout and new sewer layout will have 0 maintenance cost between the years 2024 to 2050

Investment costs for sewer pipes

The basic price for replacing a sewer with a diameter of 300 mm is €470 per m (RIONED, 2021). The basic price for replacing a sewer with a diameter of 700 mm is €940 per m. This includes the material costs, groundwork including the addition of new soil, road hardening, miscellaneous expenses (such as traffics measures) and labor costs. The cost figures for other diameters are derived from this.

The cost figures for replacing concrete sewers can be calculated using the following formulas m (RIONED, 2021):

- Cost figure for a diameter up to 700 mm and unreinforced:
basic price (300 mm) x 1.25 ^ (diameter - 300)/135 = cost figure per m

- Cost figure for a diameter of 700 mm or more and reinforced:
basic price (700 mm) x 1.17 ^ (diameter - 700)/135 = cost figure per m

Maintenance cost for sewer pipes

The annual maintenance costs for sewer pipes is calculated using the below equation developed by RIONED (Suryanto, 2021). This includes cost for high-pressure cleaning. The maintenance cost depends on the pipe diameter and the daily deposition production, approximately €1,500 per day (Li, 2020). However, the cost can be translated into € / m value with the equation below.

Annual maintenance cost = 6.15 𝑥 10−4 𝑥 1500 (𝑏𝑎𝑠𝑖𝑐 𝑝𝑟𝑖𝑐𝑒)𝑥 2.718𝐷 (𝑚𝑚) 𝑥 0.0017

Other indicators

|  | Assumed value for upgrade layout | Assumed value for new layout | Reasoning |
| --- | --- | --- | --- |
| Additional cost to connect houses to the sewer systems | 50,000 euros | 450,000 euros | Rough estimate. New layout will lead to potential ripping of house floors and laying down new pipes to connect to the sewer in the front streets |
| Monumental value | High | Low | New layout will lead to potential ripping of house floors and laying down new pipes to connect to the sewer in the front streets, thereby impact the old flooring and architecture of the house |
| Construction nuisance | Low | High | New layout will lead to potential ripping of house floors and laying down new pipes to connect to the sewer in the front streets, leading to a lot more nuisance for residents inside the house |

## B.4. SUDS performance assessment

Assumptions:

- Effective storage capacity is not taken into account and it is assumed that all SUDS would be empty before the next shower

| SUDS | Description | Water storage capacity | Investment cost | Maintenance cost | Contribution to BG area. Yes/No | Data sources |
| --- | --- | --- | --- | --- | --- | --- |
| Standard sedum roof | A shallow and lightweight layer of plants that absorbs water. It is easy to maintain and can withstand dry periods. | 30 l/m2 | 110 €/m2 | 20 €/m2 | Yes, % of area selected by player * total available roof area | (Brolsma, 2024; Natural Water Retention Measures, n.d.-a) |
| Intensive green roof | A deep planted soil layer on a roof that absorbs water. It holds larger plants, that need to be watered during dry periods | 45 l/m2 | 155 €/m2 | 60 €/m2 | Yes, % of area selected by player * total available roof area | (Natural Water Retention Measures, n.d.-a) |
| Rainwater fence | Vertical rain barrel on a fence or a building wall. During dry periods, residents can use the water collected to irrigate their gardens | 440l/unit | 250 €/unit | 50 €/unit | No, the measure doesn’t contribute to any additional blue-green area | (Kennisportaal Klimaatadaptaties, n.d.; WaterfFence, n.d.) |
| Rain barrel | Standard size rain barrel for rain collection and watering plants in the garden | 200 l/unit | 250 €/unit | 50 €/unit | No, the measure doesn’t contribute to any additional blue-green area | (Hornbach, n.d.; Natural Water Retention Measures, n.d.-d) |
| Rainwater storage tank | A storage tank under the terrace or garden to store rainwater for watering plants or reuse for household purposes. | 500 l/unit | 2500 €/unit | 1000 €/unit | No, the measure doesn’t contribute to any additional blue-green area | (Enduramaxx, n.d.; Natural Water Retention Measures, n.d.-d) |
| Permeable pavement tiles | Converting sealed and paved backyards and frontyards into an impervious surface that lets rainwater infiltrate into the ground | 30 l/m2 | 70 €/m2 | 7 €/m2 | Yes, % of area selected by player * total available outside space | (Brolsma, 2024; Natural Water Retention Measures, n.d.-b) |
| Rain garden | A shallow vegetaded area in the back or frontyard that soaks rainwater coming from the roof rain pipes. | 40 l/m2 | 150 €/m2 | 15 €/m2 | Yes, % of area selected by player * total available outside space | (Natural Water Retention Measures, n.d.-c) |
| Green facades | Plants are rooted in the soil on the ground and climb on a façade | 40 l/m2 | 800 €/m2 | 5 €/m2 | Yes, % of area selected by player * total available wall space | (Brolsma, 2024; Kew et al., 2014; Perini & Rosasco, 2013) |
| Vertical garden | A wall covered with plants that are growing in containers or on special material attached to the wall | 25 l/m2 | 500 €/m2 | 10 €/m2 | Yes, % of area selected by player * total available wall space | (Hou het warm, n.d.) |

# C. ICUD conference game play experience results

| Respondent number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | Average scores |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Statements |  |  |  |  |  |  |  |  |  |  |  |  |
| I had a good time playing this game | 5 | 6 | 6 | 6 | 7 | 5 | 6 | 5 | 6 | 5 | 5 | 5.6 |
| The game felt relevant to me | 6 | 5 | 5 | 7 | 5 | 4 | 7 | 3 | 5 | 5 | 5 | 5.2 |
| I felt I was good at playing this game | 4 | 4 | 5 | 7 | 6 | 4 | 6 | 4 | 5 | 5 | 6 | 5.1 |
| I was fully focused on the game | -- | 5 | 7 | 5 | 7 | 5 | 4 | 6 | 5 | 4 | 5 | 5.3 |
| I felt a sense of freedom about how I wanted to play this game | 5 | 5 | 6 | 6 | 7 | 5 | 4 | 5 | 6 | 5 | 4 | 5.3 |
| I felt eager to discover how the game continued | 5 | 4 | -- | 6 | 6 | 6 | 6 | 5 | 6 | 6 | 4 | 5.4 |
| The actions to control the game were clear to me | 5 | 4 | 5 | 7 | 6 | 5 | 4 | 2 | 6 | 2 | 3 | 4.5 |
| The challenges in the game were at the right level of difficulty for me | 6 | 6 | 5 | 6 | 6 | 5 | 5 | 6 | 5 | 5 | 5 | 5.5 |
| I could easily assess how I was performing in the game | 4 | 4 | 5 | 7 | 6 | 5 | 3 | 2 | 4 | 2 | 4 | 4.2 |
| I liked the look and feel of the game | 4 | 5 | -- | 7 | 7 | 6 | 5 | 3 | 7 | 2 | 5 | 5.1 |
| I grasped the overall goals of the game | 5 | 6 | -- | 7 | 5 | 4 | 7 | 5 | 7 | 3 | 4 | 5.3 |

# D Post-game questionnaire

# E Urban dRain introduction presentation

# F Game materials

## F.1. Group title headings

These can be folded along their length and kept on the tables of the two groups to distinguish them

## F.2. Player role cards

Front and back sides of each role card

## F.3. SUDS cards

# G Results from play-test sessions

## G.1. Summary of solutions chosen

Teams marked in green were declared the winners of the particular session

| Group | Layout chosen | SUDS options | Flood damage | Budget utilized | % of blue-green area added | Cost distribution |
| --- | --- | --- | --- | --- | --- | --- |
| Morning session |  |  |  |  |  |  |
| 1 | Upgrade back alley - Middle pipe: 0.3 m 0.45 m - Downstream pipe 0.4m 0.5 m | - Standard sedum roof (50%) - Intensive green roof (10%) - Rain barrels (15%) - Permeable pavements (10%) | 0 | 1485 | 36 | - MUN - 992 (SUDS investment cost – 53%, SUDS maintenance cost – 20%, Sewer investment cost – 100%, Sewer maintenance cost – 100%) - RES – 493 (SUDS investment cost – 47%, SUDS maintenance cost – 80%, Repiping cost – 100%) |
| 2 | Upgrade back alley - Middle pipe: 0.3 m 0.45 m - Downstream pipe 0.4m 0.5 m | - Permeable pavements (50%) - Rain gardens (5%) | 0 | 942 | 20 | - MUN – 598 (Sewer investment cost – 100%, Sewer maintenance cost – 100%) - RES – 343 (SUDS investment cost – 100%, SUDS maintenance cost – 100%, re-piping cost – 100%) |
| 3 | New system - Upstream pipe (0.3 m) - Middle pipe (0.4 m) - Downstream pipe (0.45 m) - End pipe (0.45 m) | - Standard sedum roof (5%) - Intensive green roof (5%) - Rainwater fence (10%) - Rain barrels (20%) - Rainwater storage tank (10%) - Permeable pavements (25%) - Rain garden (30%) | 0 | 1491 | 26 | - MUN - 995 (SUDS investment cost – 29%, Sewer investment cost – 100%, Sewer maintenance cost – 100%, Repiping cost – 100%) - RES – 496 (SUDS investment cost – 71%, SUDS maitenance cost – 100%) |
| 4 | Upgrade sewer - Middle pipe: 0.3m 0.4 m - Downstream pipe 0.4m 0.5m | - Permeable pavements (50%) - Rain garden (20%) | 0 | 1467 | 42 | - MUN - 990 (SUDS investment cost – 46%, Sewer investment cost – 100%, Sewer maintenance cost – 100%, Repiping cost – 100%) - RES – 477 (SUDS investment cost – 54%, SUDS maintenance cost – 100%) |
| Afternoon session |  |  |  |  |  |  |
| 5 | Upgrade - Middle pipe: 0.3 m 0.45 m - Downstream pipe 0.4m 0.5 m | - Standard sedum roof (20%) - Rainwater fence (30%) - Rainwater storage tank (20%) - Permeable pavements (25%) - Rain garden (10%) | 0 | 1244 | 24 | - MUN - 812 (SUDS investment cost – 32.5%, Sewer investment cost – 100%, Sewer maintenance cost – 100%, Repiping cost – 100%) - RES – 432 (SUDS investment cost – 67.5%, SUDS maintenance cost – 100%) |
| 6 | Upgrade - Middle pipe: 0.3 m 0.4 m - Downstream pipe 0.4m 0.45 m | - Standard sedum roof (50%) - Permeable pavements (50%) | 0 | 1464 | 45 | - MUN - 881 (Sewer investment cost – 100%, Sewer maintenance cost – 100%) - RES – 584 (SUDS investment cost – 100%, SUDS maintenance cost – 100%, Repiping cost – 100%) |
| 7 | Upgrade - Middle pipe: 0.3 m 0.35 m - Downstream pipe 0.4m 0.5 m | - Standard sedum roof (50%) - Permeable pavements (50%) - Rain garden (5%) | 0 | 1493 | 47 | - MUN - 995 (SUDS investment cost – 67%, SUDS maitenance cost – 67%, Sewer investment cost – 67%, Sewer maintenance cost – 67%, Repiping cost – 67%) - RES – 498 (SUDS investment cost – 33%, SUDS maitenance cost – 33%, Sewer investment cost – 33%, Sewer maintenance cost – 33%, Repiping cost – 33%) |
| 8 | Upgrade - Middle pipe: 0.3 m 0.5 m - Downstream pipe 0.4m 0.5 m | - Standard sedum roof (10%) - Rainwater storage tank (20%) - Permeable pavements (40%) | 0 | 1139 | 20 | - MUN - 656 (Sewer investment cost – 100%, Sewer maintenance cost – 100%) - RES – 484 (SUDS investment cost – 100%, SUDS maintenance cost – 100%, Repiping cost – 100%) |

## G.2. Response to Q1: How do you currently feel about the Urban dRain game session?

## G.3. Response to Q2: What makes you feel this way?

| No. | Response | Theme |
| --- | --- | --- |
| 1 | It was a nice case, but the sheet had some weird issues with solutions increasing flood damage. | Actions |
| 2 | it was a bit messy, as the excel did not work really good and it was not clear what we needed to do | Actions |
| 3 | The sheet was a bit unclear | Actions |
| 4 | Theres a problem with the excel formula | Actions |
| 5 | The excel did not work | Actions |
| 6 | It was a fun process just the terms were a bit unclear. We were not aware that we could have transferred our remaining budget to the residents | Actions; rules |
| 7 | Nice game, but feel that like a lot of factors didn't matter. Also thé excel sheet was quite hard to work with | Actions; rules |
| 8 | The vague explanation of the exercise | Actions; rules; design |
| 9 | It was a good game but i didnt understand the underlying thought of what we should learn from it. Was it about the numbers or negotiation? | Actions; rules; design |
| 10 | Bit overcomplicated | Actions; rules; design |
| 11 | To me it’s weird that your score is greatly dependent on your blue green card area, while there are a lot of actors who want to take into account other things. This means that you can’t just look at a single critical element to call a winner. We chose the new layout, but an upgrade would have been cheaper. If we chose the second option and invested more in blue green area, our outcome would have been better judging purely on that criteria. The maintenance costs for example weren’t taken into account when determining which teams performed better. | Actions; rules; design |
| 12 | It’s easy to follow and understand. An intuitive way to simulate real life government-resident interaction. | Actions; rules; design |
| 13 | I liked it a lot but some of the initial instructions weren’t clear | Rules; design |
| 14 | I think the budget was way too high which wasn’t realistic | Rules; design |
| 15 | It was interesting to make an optimization in the excel for the residents. However, it was not entirely clear to me that the budget of the municipality could be fully used, so we did not achieve max blue green percentage | Rules; design |
| 16 | Needs some fixing in the usability, and experience can be more playful than an excel with fixed calculation -> resulting in the possibility of finding the actual and optimal spending split, thus reducing the game to a trial-and-error escapade | Rules; design |
| 17 | I wasn't sure about our roles in the game in round 1 and 2 and the objectives. | Rules; design |
| 18 | I like the idea, but it would be nice to have actual boards with information (like board game material) instead of loose papers | Design |
| 19 | It didn't really feel like a game, more like a study assignment. A fun assignment! | Design |
| 20 | i think the assignment wasn’t explained that well, so the not TB people didn’t really understand it. They are not that fimiliar yet with the idea of multiple actors with different goals and that you have to find a solution that meets all the goals. I had to explain it all en that took a while | Design |
| 21 | It wasn't really a game. It was optimalizing with trial and error, while I know that a computer could do this in a few seconds. With a for instance Monte Carlos simulation. | Design |
| 22 | I enjoyed the case although missed a game element and a more mathematical optimising method. | Design |
| 23 | I liked the numbers | Design |
| 24 | My group won. | Challenge |
| 25 | I didn’t achieve my goals | Challenge |
| 26 | I really like the case subject! Easy to follow and understand the different perspectives. The excel sheet was simple and easy to use. I feel like the case could be more challenger and fun by making the requirements harder to reach. So the solutions become more diverse! | Challenge; good time |
| 27 | It was fun, mostly start with only residents | Good time |
| 28 | It was fun | Good time |
| 29 | I like numbers | Good time, eagerness |
| 30 | We had a good time experimenting | Good time, eagerness |
| 31 | It was quite fun but i missed the challenge of the game | Good time; challenge |
| 32 | I can imagine how having this tool in my hand will help making decision. | Relevance |
| 33 | its a fun way to solve a serious problem | Relevance, good time |
| 34 | New, therefore suprising. A lot of different options to play, nice combination of prescribed/formal restrictions an freedom to negotiate. | Relevance, good time; challenge |
| 35 | I liked the excels, but I missed some critical thinking, cause you could just fill in some numbers | Relevance; goals |
| 36 | It was interesting to see what parameters changed the outcome. Also it was interesting to see when the 2 parties came together | Relevance; good time |
| 37 | It was fun, it felt like a realistic/real case | Relevance; good time |
| 38 | Good way to learn something about it in a playful way | Relevance; good time |
| 39 | Fun case to work on, nice measures and options to take. However, the set-up with these requirements didn't seem like a wicked game. Even though there were different parties, their wants and needs aligned in the optimal case. | Relevance; good time |

## G.4. Response to Q3: How strongly do you agree or disagree with the following?

## G.5. Response to Q4: What is your educational background/ what bachelor's degree did you pursue? (e.g. economics, civil engineering, so on)

## G.6. Responses to Q5, Q6, and Q7: Group no., player role, and goals prioritized

## G.8. Response to Q8: How strongly do you agree or disagree with the following?

## G.9. Response to Q9: Was there anything remarkable or unexpected that you are taking away from this session? If so, what?

| No. | Response |
| --- | --- |
| 1 | Not really |
| 2 | It was fun and learned a lot about the collaborative aspect with residents |
| 3 | No |
| 4 | Being transparent and reaching a common goal works |
| 5 | There were more players on our team, which meant that there were more citizens with the same role. This made negotiations harder. |
| 6 | One side of the table is almost always gonna be blindfolded by what the other side has in their agenda |
| 7 | No |
| 8 | No |
| 9 | How many different solutions to the problem at hand is |
| 10 | the size of the budgets |
| 11 | Good negotiations were not assessed |

## G.10 Response to Q10:  Please share any additional comments, suggestions or feedback you have for the research team.

| No. | Response |
| --- | --- |
| 1 | The goal was clear but felt unrealistic, everyone had the feeling they should not maximise spending but that was an easy way to win, the factors other than blue/green and money were not taken into account in the solution criteria or made measurable |
| 2 | I did not really understand why the different roles and wishes of the residents and municipality were added. For example a frugal resident. In the end, you don’t win the case on how well you negotiated, but just how high your green % is, so the frugal resident has no incentive to really debate for lower costs. |
| 3 | Thé icons on thé Paper of thé different options didn't match thé values in thé excel sheet. So that was a bit confusing. I also felt like in the end we didn't look at all thé values anymore but just tried to get maximum Blue green and stay just under budget. (So we kind of won but not in the right way). There was no incentive for negotiation |
| 4 | I think it would make more sense to include more factors in first sheet of the municipality because they have more insentives than only flood damage. Maybe a transfer money possibility aswell: a way that the municpality can cover costs for the residents |
| 5 | The group size could maybe be a little smaller. |
| 6 | - |
| 7 | Decrease the importance of the excel sheet and make the negotiations more important for the final solutions. A good pitch in the end would make it nicer as well:) |
| 8 | Having some issues with the tables on different sheets because they’re not linked correctly. Other than that, it’s perfect! |
| 9 | i like it better when you have to think of possible solutions yourself instead of them already being presented like this (the green- blue solutions) |
| 10 | Nvt |
| 11 | The busget was too high so it was quote easy. And the excel had mistakes in it, which meant I lost interest and motivation |
| 12 | The calculations behind the effect of rainbarrels doesnt work correctly |
| 13 | / |
| 14 | I think how the game is played needs to be revamped as it doesn't quite feel like a game yet, albeit with good educational points being conveyed |
| 15 | It would be great to have the rules/ steps/ objectives written down. It would be good to see at one glance which cells need to be filled in the excel sheet and which update automatically |
| 16 | A lot of things were just not adding anything. Like the rain barrels. And quickly we dropped all roles and just tried to optimalize. |
| 17 | I would find a wat to 1. Assess the negotiation part 2. Optimise the allocation of money in a more thoughtfull way, not only by trail and error |
| 18 | Maybe add a subjective value (still in numbers), like the nicer green roof looks 2 better than the other green roof, but still costs more. In that sense you get choose a point of view on which you optimise, then let the teams present their choices and choose a winner by voting |

## G.11. Debriefing notes

- How was it to step into the shoes of the municipality/residents and how can you relate it to the real-world?

Group 3: Like learning about the different options and their effects; relates to the real world as people cannot make their own plans – there is no platform to bring the sperate solutions together

Group 4: Blue-green area should works as a balance (?), it should not just be one factor, would recommend adding a diversity the solution space

Group 2: Interesting to see that it is difficult to not only thing about the minimizing of the cost; that in this case it might be better to spend more to get a better solution

Group 1; residents got their way

Group 5; did not really get into the sin of the stakeholder, as the group goal was also in the back of their mind, not really incentive to get into the individual role as they were thinking about the team role; wining as a team was more important

- Reflecting on the criteria to get to a winner should be reflected upon; especially the high budget

Group 6; new is to much nuisance

Group 5; new would be too expensive, did not get that they could redistribute the budget, should be made clearer – futureproof is not really considered now as it is not a fixed parameter something was not moving along with the other values which made playing the game a little bit hard to understand

- Was there anything challenging about the decision-making? How did you go about improving the solution?

Group 4; Not a lot of ways to make it better, only to make it greener  used the remaining budget to make it more green

Group 3; focused on making a village for the future, this solution had the highest resilience for the future – presented two basis option for the basic upgrade and the new system, but the residents seemed to lean on the new one based on the flooding damage, so they kept working on that one and gave them more money for the green-blue solution. Residents were okay with the nuisance, completely covered by (?), in the real world it will create conflict, but if you give them the information of why you are doing it and why it is important for the future it helps; also the advantage of having all of the residents around the table as opposed to in real life, so you can convince them

Group 7; lot of tweaking numbers to get to the results which was somewhat difficult to find out the percentages of the different groups; separately came to the same answer- manly optimised without taking the roles into account – individual plans will change when working together which was interesting to see (residents wanted more green, mun had a lot of money left which was used to get more greens, which in turn decreased the need for thicker pipes) – especially for blue-green the roofs was very much the optimal thing, might be nice to get a maximum there to get some diversity there, how it is values might be nice to change as right now they just looked at what could give the most points and went for that – silo opinion of the residents was completely ignored as that did not get enough points – maybe add afterwards that the stakeholders can vote if they are satisfied with the final solution (contentment level, greenroofs might look nicer, which while it is expensive, might increase the satisfaction with the solution)

- Was there anything remarkable or unexpected that you are taking away from this session? If so, what?

Group 3; quite liked that everybody had their own role, quite interesting for the conversation – they had more fight within the municipality about the cost effectiveness, they gave into the engineers because of the citizens well being (one guy turned around for a second and everything became about costs)

Group 1; round 2 municipality thought they had the right solution before discussing it with the residents, only after the discussion with the residents they changed their decision

Group 4; large incentive to only to green-blue solution, as opposed to also looking at the budget a lot as they all agree on having a lot of green-blue  try to either making the budget lower or get every player to have their own budget and they have to pool in the budget together, if they don’t agree they wont get the investment – compare the two numbers, how does green space compare per the amount of money that it costs; getting a balance here is hard, they first made the sewers to meet the criteria and then maximized the greenspace (but they did that with the entire budget)

Group 7; optimization with trial and error, would have been more fun to get mathematical ways for optimization – it is fun to see the different options and the different opinions of the actors, and that they do actually impact each other

# H Final prototype iteration

## H.1. Introducton presentation

## H.2. Role cards

## H.3. Excel sheet images

## H.4. Play-test results

### Response to Q1: How do you currently feel about the Urban dRain game session?

### Response to Q2: What makes you feel this way?

| No. | Response |
| --- | --- |
| 1 | I think it was interesting to play and see the effect of the different solutions |
| 2 | Open and safe environment. Similar objectives within resident group. |
| 3 | Lots of fun, with some stereotyping. Couldn't really read the excel and felt some information was missing, eg on long-term effects |
| 4 | It was a very interesting game session. |
| 5 | Interactive and competent environment |
| 6 | It was an interesting and fun exercise, but I also felt slightly restricted by the strict preformatted options combined with limited time to explore the full range to find the optimum. |

### Response to Q3: How strongly do you agree or disagree with the following?

### Response to Q4: What is your educational  background/ what bachelor’s degree did you pursue?

| No. | Response |
| --- | --- |
| 1 | Civil Engineering |
| 2 | Landscape architecture and civil engineering |
| 3 | PhD |
| 4 | Political and Behavioral Science |
| 5 | Civil engineering |
| 6 | Engineering and policy analysis |
| 7 | Civil engineering |

### Response to Q6 and Q7: player role and goals prioritized

### Response to Q8: How strongly do you agree or disagree with the following?

### Response to Q9: Was there anything remarkable or unexpected that you are taking away from this session? If so, what?

| No. | Response |
| --- | --- |
| 1 | None, all parts were valuable |
| 2 | That urban NBS are insufficiënt for rainfall. However, only grey would do. So there is a steong need to make the multi-value visible, |
| 3 | I felt there were max four options to discuss that we could have checked from the start to save some time in the discussion as some measures were not having much effect in the flood and biodiversity goals. |

### Response to Q10: Please share any additional comments, suggestions or feedback you have for the research team.

| No. | Response |
| --- | --- |
| 1 | May be review a bit the benefits of the bgi solutions |
| 2 | It would be good to emphasize from the start that the actors on the municipality can form a coalition with the residents and do not have to stick with the municipality position. This gives the actors with less financial resources more leverage in the first round. |

# References

Brolsma, R. (2024). Tables construction and maintenance cost (Dutch prices).

Butler, D., & Davies, J. (2011). Urban Drainage (3rd ed.). CRC Press. https://doi.org/10.1201/9781315272535

Enduramaxx. (n.d.). 5,000 Underground Rainwater Tank. Retrieved December 4, 2024, from https://enduramaxx.co.uk/enduramaxx/5000-litre-1100-gallon-underground-water-tank/

Hornbach. (n.d.). Regenton (rain barrel) Whisky 190 L. Retrieved December 4, 2024, from https://www.hornbach.nl/p/regenton-whisky-190-l/10153267/?wt_mc=nl.paid.sea.google.alwayson_assortment.gar.product.17430310680.138210625180.&gclid=CjwKCAjwgZCoBhBnEiwAz35Rwixb6uX_3l9tW6JR4ny2bRU2oKOoVtx3cdrT7A_sHlkEtGEwdRpiHRoC80IQAvD_BwE

Kennisportaal Klimaatadaptaties. (n.d.). Rainwater fence in Zwolle collects rainwater and encourages greening - Spatial adaptation. Retrieved December 4, 2024, from https://klimaatadaptatienederland.nl/en/@215615/rainwater-fence-zwolle-collects-rainwater/

Kew, B., Pennypacker, E., & Echols, S. (2014). CAN GREENWALLS CONTRIBUTE TO STORMWATER MANAGEMENT? A STUDY OF CISTERN STORAGE GREENWALL FIRST FLUSH CAPTURE. Journal of Green Building, 9(3), 85–99. https://doi.org/10.3992/1943-4618-9.3.85

Li, J. (2020). A data-driven improved fuzzy logic control optimization-simulation tool for reducing flooding volume at downstream urban drainage systems. Science of The Total Environment, 732, 138931. https://doi.org/10.1016/j.scitotenv.2020.138931

Natural Water Retention Measures. (n.d.-a). Green Roofs. Retrieved December 4, 2024, from https://www.nwrm.eu/measure/green-roofs

Natural Water Retention Measures. (n.d.-b). Permeable surfaces. Retrieved December 4, 2024, from https://www.nwrm.eu/measure/permeable-surfaces

Natural Water Retention Measures. (n.d.-c). Rain Gardens. Retrieved December 4, 2024, from https://www.nwrm.eu/measure/rain-gardens

Natural Water Retention Measures. (n.d.-d). Rainwater Harvesting. Retrieved December 4, 2024, from https://www.nwrm.eu/measure/rainwater-harvesting

Perini, K., & Rosasco, P. (2013). Cost–benefit analysis for green façades and living wall systems. Building and Environment, 70, 110–121. https://doi.org/10.1016/j.buildenv.2013.08.012

Pötz, H., & Bleuzé, P. (2012). Urban blue-green grids for sustainable and dynamic cities. coop for life.

RIONED. (2019). Hydrogrammen bui07 and bui0 (T=2). https://www.riool.net/kennisbank/onderzoek/modelleren-hydraulisch-functioneren/stap-iv-hydraulische-belasting-bepalen/hemelwaterafvoer-hwa/ontwerpbuien-met-statistische-herhalingstijd/bui01-bui10/hydrogrammen-bui07-en-bui08-t-2

RIONED. (2020). Hydrogrammen composietbuien scenario’s 2030. https://www.riool.net/kennisbank/onderzoek/modelleren-hydraulisch-functioneren/stap-iv-hydraulische-belasting-bepalen/hemelwaterafvoer-hwa/ontwerpbuien-met-statistische-herhalingstijd/composietbuien/composietbuien-voor-klimaatscenarios/hydrogrammen-composietbuien-scenarios-2030

RIONED. (2021). Vrijvervalriool - vervanging (Gravity sewer - replacement). https://www.riool.net/kennisbank/financien-gemeentelijke-watertaken/kostenkengetallen/kostenkengetallen-per-rioleringsobject/vrijvervalriool/vrijvervalriool-vervanging

Suryanto, A. S. (2021). Optimal Rehabilitation of Urban Drainage Systems [Delft University of Technology]. https://repository.tudelft.nl/record/uuid:2c9aef02-d0b7-4973-bf1f-ccda469b9616

Vogelzang, L. M. (2023). An assessment framework for SUDS-based storm water management in urban areas [MSc. thesis] [Delft University of Technology]. https://repository.tudelft.nl/record/uuid:57468ca6-b7f5-4623-9e52-d9ddf5e98b50

WaterfFence. (n.d.). Innovative Rainwater And Greywater Harvesting System For Commercial And Residential Use. Retrieved December 4, 2024, from https://waterfence.com/

Hou het warm. (n.d.). What are the costs of a vertical garden. Do it yourself or have it done? Retrieved December 4, 2024, from https://houhetwarm.nl/verticaletuin/verticale-tuinen/kosten-verticale-tuin/