BLUESKY OPEN ATM SIMULATOR
 COMMAND REFERENCE

HELP FUNCTION

?	Use  ?  as (only) in-line argument to read helptext and argument list or type
command without arguments

SPECIAL KEYS

ESC/Close window			Quit program

F3-key					Recall last typed command on edit line of console window

F11-key					Toggle full-screen mode

Plus/minus				Zoom in/out (“=”interpreted as “+” as it is on the same key)

Cursor keys/Numeric keypad arrows	Pan left.right,up,down

Drag/drop  console edit window and quick-menu window to move it

TRAFFIC COMMANDS

CRE acid, type, lat, lon, hdg, alt, spd	Create an aircraft at specified position (use mouse)

DEL acid				Deletes an aircraft

MOVE acid,lat,lon,alt			Move an aircraft (use mouse)

(acid) or POS acid	Retrieves position, route & info on aircraft (double click a/c = POS),
double click a/c (POS) twice to make route disappear again

(acid) HDG (hdg)		Heading command

(acid) LEFT/RIGHT (delhdg)	Relative heading command

(acid) SPD (IAS/Mach)		Speed command

(acid) ALT (alt) [,vertspd]	Altitude command (optional with vertical speed)

(acid) VS (vertspd)		Vertical speed (first set commanded altitude)

TRAFFIC NAVIGATION COMMANDS

(acid) DEST (airportid)		Set destination for navigation purposes

(acid) ORIG (airportid)		Set origin (for bookkeeping purposes)

(acid) ADDWPT (waypoint/lat,lon), (alt), (IAS/Mach) [, (afterwp)	Add a waypoint or lat/lon position to route at the end before the destination or insert after a afterwp (if this is the 1st waypoint, make active and switch on LNAV)

(acid) DELWPT (waypointid)	Delete a waypoint using the id given by LISTRTE

(acid) LISTRTE [pagenr]	Use LISTRTE for page 0, pagenr used by program ot display 7 wpts/page

{acid} LNAV ON		Switch LNAV mode on

{acid} VNAV ON		Switch VNAV mode on

SIMULATION CONTROL

IC [playfile],[recfile]			Initialize condition, just IC with ni arguments opens the Dialog Box

OP/START/RUN/CONTINUE		Start or continue running

HOLD/PAUSE				Pause or hold simulation

EXIT/QUIT/Q/STOP/END		Exit program (or use ESC key)

SAVEIC filename			Save current situation as IC

DT [dt]	Sets time step to the value dt for FIXDT, shows current DT without argument

FIXDT ON/OFF[,forhowmanyseconds]	Forces the Traffic and Experiment Manager to use a fixed time step

DISPLAY COMMANDS

ZOOM IN/OUT		Set zoom of current display (radar or navigation display)

++++ / -----		Multiple zoom in (+) or zoom out (-), “=” is interpreted as “+” as it is on the same key

PAN (LEFT / RIGHT / UP / DOWN / RFS / MCS / acid / airport / lat, lon ) Pan radar window

SWRAD GEO / SAT/ GRID / APT / VOR / WPT / NDB / LABEL toggles features on or off

MISCELLANEOUS COMMANDS

DIST lata,lona,latb,lonb			Calculate bearing and distance from A to B

CALC expression			Calculator e.g. 5*sin(2.3) 120*NM (use NM, KTS, FT for units)

METRICS OFF/0/1/2 [dt]		Calculate & show traffic complexity metrics for AREA

AREA lata,lona,latb,lonb[,lowalt]	Specify experiment area (leaving a/c are deleted)

AREA OFF				Switch experiment area off

TAXI ON/OFF	Switch taxi option on/off. OFF = traffic descending through 1500 ft will be deleted