Wednesday morning, breakwater in St. Konstantin

Tetrapod damage

| Damage level | Breakwater head (largest tetrapods) | Other part (smaller tetrapods) |
| --- | --- | --- |
| More or less undamaged | 9 | 30 |
| Minor cracks | 4 | 11 |
| Major cracks | 7 | 5 |
| Broken | 3 (and 1 split) | - |

From visual observation. Not all tetrapods included.

Number of tetrapods

Last 5 slabs:

20 on each of a concrete slab

Middle 5 slabs:

15 each side of a concrete slab

First 9 slabs:

10 each side of a concrete slab

Head: 35

North

Dimensions of breakwater

Length of crest 95 m

Crest width 8,60 m

Width of concrete slab: 5 m

Height of crest above still water line: 1,70 m

Slope above still water line: 1.5 m vertical : 1 m horizontal